# OpenShift Virtualization vs VMware: Key Differences, Technologies & Use Cases

Table of Contents

1.	Understanding Virtualization Technology	4

Core Concepts of Virtualization	4

The Role of Hypervisors	4

Type 1 Hypervisors	4

Type 2 Hypervisors	4

Resource Management in Virtual Environments	5

Real-World Example:	5

1. CPU Scheduling	5

2. Memory Management	5

3. Storage Virtualization	6

4. Network Resource Sharing and Isolation	6

Simple Real-Life Analogy	6

Exam-Friendly Definition	7

VMware's Enterprise Virtualization Platform	7

ESXi: The Foundation of VMware Infrastructure	7

2.	OpenShift Virtualization with KubeVirt	7

Merging Container and VM Workloads	7

KubeVirt Technology	7

KVM Integration	7

3.	Hardware & Storage	8

HPE iLO and Dell iDRAC	8

What is HPE iLO?	8

What is Dell iDRAC?	10

The most important concept: Out-of-Band Management	11

In-Band Management	11

Out-of-Band Management	12

Real-World Example 1: — The Server Is Hung	13

Real-World Example 2 — Windows Server Crash	13

Hardware Health Monitoring	14

Real-World Example — Fan Failure	14

Temperature monitoring	14

RAM/DIMM monitoring	15

Storage aur RAID monitoring	15

HPE Storage Architecture	15

Dell Storage Architecture	16

RAID monitoring	16

Real-World RAID Example	16

Power Supply Monitoring	17

Virtual Media — very important	17

Firmware Updates	18

Logs — A Critical Part of Troubleshooting	18

Dedicated IP Addresses for iLO and iDRAC	19

Management Network	19

VMware with iLO / iDRAC	20

Out-of-Band Remediation via iLO:	20

Activity 1: Data Centre Components	21

Rack Manifold	21

CDU	21

How to Connect a CDU, Rack Manifold, and GPU Servers	22

Step-by-step connection process	24

Step 1: Connect the CDU to the cooling infrastructure	24

Step 2: Connect the CDU to the rack manifold	24

Step 3: Connect the rack manifold to the GPU servers	24

CDU (Racks)	25

Liquid cooling hoses	25

Firmware	25

What is a RAID Controller?	27

RAID 0 - Performance	27

RAID 1 - Mirroring	28

RAID 5 - Performance + Protection	30

How is Data Recovered When a Drive Fails? (Data Reconstruction)	30

Difference between Firmware and RAID Controller?	31

RAID 10 (also known as RAID 1+0	31

How Does It Work? (Mirroring + Striping)	31

How is Data Protected When Drives Fail?	32

Direct Comparison: RAID 10 vs. RAID 5	32

Storage	33

SAN vs NAS	33

What is NAS?	33

Fig. Network Attach Storage	34

Which protocols are used in NAS?	35

Where is NAS used?	35

File vs. block vs. object storage	35

What is SAN?	35

SAN vs NAS important difference	38

# Understanding Virtualization Technology

## Core Concepts of Virtualization

Virtualization technology enables multiple operating systems to operate independently on a single physical server. This technology creates isolated software environments that share underlying hardware resources efficiently. At its core, virtualization transforms physical computing components into software-defined resources, allowing for better resource utilization and increased operational flexibility.

## The Role of Hypervisors

The hypervisor serves as the foundation of virtualization technology, acting as the control center that manages and distributes hardware resources among virtual machines. This essential software layer creates and maintains separation between physical hardware and virtual environments, ensuring secure and efficient operation of multiple virtual instances.

## Type 1 Hypervisors

Bare-metal hypervisors, known as Type 1, represent the most efficient virtualization approach. These hypervisors install directly on server hardware, eliminating the need for a host operating system. This direct hardware access results in superior performance and reduced resource overhead. Notable examples include VMware ESXi, KVM, and Citrix Hypervisor. Organizations typically choose Type 1 hypervisors for production environments where performance and security are paramount.

## Type 2 Hypervisors

In contrast, Type 2 hypervisors operate as applications within a conventional operating system. While these hosted hypervisors offer easier installation and management, they introduce additional overhead due to the underlying operating system layer. Solutions like VMware Workstation and Oracle VirtualBox fall into this category. These hypervisors excel in development, testing, and desktop virtualization scenarios where maximum performance isn't critical.

## Resource Management in Virtual Environments

Successful virtualization depends on efficient resource allocation and management. Modern hypervisors employ sophisticated techniques to distribute computing resources, including:

- CPU scheduling and allocation

- Memory management and distribution

- Storage virtualization and allocation

- Network resource sharing and isolation

These resource management capabilities enable organizations to maximize hardware utilization while maintaining performance and isolation between virtual instances. The hypervisor continuously monitors and adjusts resource allocation to ensure optimal performance across all virtual machines.

### Real-World Example:

Imagine a company has a powerful physical server. Instead of using this server for only one application, a hypervisor divides it into multiple Virtual Machines (VMs).

For example, the physical server has:

- 16 CPU cores

- 64 GB RAM

- 2 TB storage

- 10 Gbps network

The hypervisor distributes these resources among different VMs:

| Virtual Machine | CPU | RAM | Storage | Use |
| --- | --- | --- | --- | --- |
| VM 1 | 4 cores | 16 GB | 500 GB | Web Server |
| VM 2 | 4 cores | 16 GB | 500 GB | Database |
| VM 3 | 2 cores | 8 GB | 300 GB | Application |
| VM 4 | 2 cores | 8 GB | 300 GB | Testing |

### 1. CPU Scheduling

Suppose VM 1 is running a website and suddenly many users visit the website. It needs more CPU power.

The hypervisor manages the CPU and gives more processing time to VM 1 when needed.

In simple words:
The hypervisor makes sure that CPU resources are not wasted and are given to the VMs that need them.

### 2. Memory Management

Suppose VM 3 has 8 GB RAM allocated to it, but it is currently using only 4 GB. Another VM may need more memory.

The hypervisor can manage the available memory efficiently according to its configuration and memory-management techniques.

In simple words:
The hypervisor helps make better use of the available RAM.

### 3. Storage Virtualization

The physical server has 2 TB of storage, but each VM appears to have its own virtual disk.

For example:

- VM 1 → 500 GB

- VM 2 → 500 GB

- VM 3 → 300 GB

The physical storage is divided and managed logically for different VMs.

In simple words:
Each VM gets its own virtual storage while the physical storage is managed by the underlying system.

### 4. Network Resource Sharing and Isolation

All VMs can share the same physical network hardware, but the hypervisor and network virtualization mechanisms manage their network traffic.

For example:

- VM 1 → Web traffic

- VM 2 → Database traffic

- VM 3 → Internal application traffic

The network resources can be controlled so that heavy traffic from one VM does not unnecessarily affect other VMs.

### Simple Real-Life Analogy

Think of a physical server as an apartment building.

- Physical Server = Apartment Building

- Hypervisor = Building Manager

- VMs = Different Apartments

- CPU = Electricity

- Memory = Water supply

- Storage = Storage space

- Network = Internet connection

The building manager distributes resources among different apartments and keeps the apartments separate.

### Exam-Friendly Definition

A hypervisor acts like a building manager. It efficiently distributes the physical server's CPU, memory, storage, and network resources among different virtual machines while maintaining isolation between them.

## VMware's Enterprise Virtualization Platform

### ESXi: The Foundation of VMware Infrastructure

VMware's ESXi represents the cornerstone of enterprise virtualization, operating as a bare-metal hypervisor that directly manages hardware resources. Unlike traditional operating systems, ESXi's streamlined architecture minimizes system overhead while maximizing performance. This efficient design enables organizations to run numerous virtual machines on a single physical server with optimal resource utilization.

- Virtual Machines virtualize the hardware, giving you an entirely separate, heavy virtual computer with its own complete operating system.

- Containers virtualize the operating system, sharing the host machine's existing kernel to run applications as lightweight, isolated processes.

# OpenShift Virtualization with KubeVirt

## Merging Container and VM Workloads

OpenShift Virtualization is a technology that allows organizations to run and manage virtual machines (VMs) and containers on the same Kubernetes-based OpenShift platform. It helps companies run old and new applications together.

## KubeVirt Technology

KubeVirt is an open-source technology that extends Kubernetes to run and manage virtual machines alongside containers.

## KVM Integration

KVM is the hypervisor used by OpenShift Virtualization to run virtual machines efficiently, while KubeVirt allows Kubernetes to manage those VMs.

We can use OpenShift Virtualization because it provides a unified Kubernetes-based platform where we can manage existing VMs while also running containerized cloud-native applications, which can simplify gradual application modernization.

# Hardware & Storage

## HPE iLO and Dell iDRAC

HPE-iLO or Dell iDARC both are remote management controller of a server

For example:

If server is a car then HPE-iLO or Dell iDARC are the remote control + dashboard + diagnostic system

You can check a server's power status, hardware health, temperatures, fans, RAM, CPU, storage, RAID controller, power supply, BIOS/UEFI, and hardware logs without having to visit it in person.

You can also perform several operations remotely, such as powering the server on or off, restarting it, launching a remote console, accessing the BIOS, installing the operating system, applying firmware updates, and mounting virtual ISOs.

### What is HPE iLO?

iLO stands for Integrated Lights-Out. It is a dedicated remote management controller embedded directly onto the motherboard of HPE ProLiant servers (such as the HPE ProLiant DL380).

Figure. HPE iLO

HPE ProLiant DL380

|

└── HPE iLO (Dedicated Hardware Chip)

HPE ProLiant DL380 is an industry-standard 2U, dual-socket rack server originally introduced by Compaq in January 2000 and currently produced by Hewlett Packard Enterprise (HPE). Known as the flagship of the "Density Line" (DL) family, it is widely regarded as a data center workhorse for enterprise virtualization, database management, cloud computing, and AI workloads.

Figure. HPE ProLiant DL380

iLO is a dedicated management controller on the server's motherboard.

Its most important feature is that:

iLO is independent of the operating system.

This means that even if Windows or Linux crashes on the server, iLO can normally still be accessed.

### What is Dell iDRAC?

iDRAC stands for Integrated Dell Remote Access Controller. It is embedded in Dell PowerEdge servers (such as the Dell PowerEdge R760).

Figure . PowerEdge 17th generation server iDRAC Direct port location

Dell PowerEdge R760

|

└── Dell iDRAC

The Dell PowerEdge R760 is a powerful 2U dual-socket rack server designed for enterprise workloads like virtualization, databases, and AI.

Figure. PowerEdge R760 Server

The purpose of iDRAC is identical to that of iLO, enabling comprehensive remote monitoring, remote power management, and remote console access. It also handles hardware monitoring, BIOS and firmware management, system logs, storage and RAID monitoring, and virtual media redirection.

| HPE | Dell |
| --- | --- |
| iLO | iDRAC |
| ProLiant | PowerEdge |
| Smart Array | PERC |
| HPE One View | Dell Open Manage |

## The most important concept: Out-of-Band Management

This is a highly critical concept for every System Engineer.

To understand it, you need to understand the two common types of management:

### In-Band Management

In this method, we manage the server through its operating system.

Administrator

↓

Network

↓

Windows/Linux

↓

Server

Examples:

- SSH

- RDP

- PowerShell

- Linux commands

The problem is, if the OS itself goes down:

Linux

↓

CRASH ❌

↓

SSH ❌

Then you don’t access server with the help of SSH.

### Out-of-Band Management

This is where the concept of iLO and iDRAC comes into play.

Administrator

↓

Management Network

↓

iLO / iDRAC

↓

Physical Server

It manages the server from outside the operating system.

Therefore:

iLO and iDRAC are classified as Out-of-Band Management technologies.

### Real-World Example 1: — The Server Is Hung

Imagine you are a System Engineer in a data center. You have an HPE ProLiant DL380 server with VMware ESXi installed.

One day, the VMware host stops responding. You try the following troubleshooting steps:

- SSH → ❌ (Fails)

- vCenter → ❌ (Shows "Host Not Responding")

- Ping → ❌ (No response)

You are not physically near the server. What do you do next?

You use iLO (Integrated Lights-Out). You log into iLO via this connection path:
Laptop → Management Network → HPE iLO → DL380 Server

Once logged in, you check the hardware status:

- Power Status: ON

- Temperature: Normal

- Fans: Normal

- Memory: Normal

- CPU: Normal

- PSU (Power Supply Unit): Normal

Since the hardware looks fine, you open the Remote Console to view the server's actual screen. On the screen, you might see a VMware ESXi Kernel Panic (Purple Screen of Death) or notice that the server is stuck on boot.

Using iLO, you perform a remote reboot. The server restarts through the following sequence:
POST → BIOS → ESXi → VMware Host → vCenter

The server successfully recovers, and you did not have to physically visit the data center.

### Real-World Example 2 — Windows Server Crash

Suppose you have a Dell PowerEdge server running Windows Server, which hosts an application.

Dell PowerEdge → Windows Server → Application

Suddenly, the application crashes, and Windows goes into a blue screen. You attempt to connect remotely:

- RDP → ❌ (Fails)

- SSH → (Obviously not available)

However, your remote management access is working:

- iDRAC → ✅ (Available)

You log into iDRAC and open the Remote Console. On the server's screen, you see the Windows Blue Screen (BSOD).

Using the iDRAC interface, you remotely restart the server to bring it back online.

## Hardware Health Monitoring

You can think of iLO / iDRAC as the server's health dashboard.

Suppose the server displays the following status:

- CPU

  - CPU 1 → OK

  - CPU 2 → OK

- Memory

  - DIMM 1 → OK

  - DIMM 2 → OK

  - DIMM 3 → Failed ❌

- Fans

  - Fan 1 → OK

  - Fan 2 → OK

  - Fan 3 → Failed ❌

- PSU

  - PSU 1 → OK

  - PSU 2 → Failed ❌

- Temperature

  - CPU Temperature → Normal

  - System Temperature → Normal

This critical information is incredibly useful for a System Engineer during the troubleshooting process.

### Real-World Example — Fan Failure

Imagine a fan fails inside a Dell PowerEdge server.

Without proper monitoring, the following chain reaction occurs:
Fan Failure → Temperature increases → CPU throttling → Performance decreases → Server shutdown

However, with iDRAC monitoring, the engineer is alerted immediately:
iDRAC → Fan 3 → FAILED ⚠️

Because of this, the System Engineer catches the issue early before any downtime happens. The hardware team can then schedule a quick visit to replace the faulty fan.

## Temperature monitoring

Server room mein cooling bohat important hoti hai.

iLO/iDRAC sensors ke through temperatures monitor kiye ja sakte hain.

Example:

CPU 1: 55°C

CPU 2: 57°C

Memory: Normal

System Board: Normal

Agar temperature unusually increase kare:

Temperature

↓

High

↓

Warning

↓

System Engineer

Potential reasons:

- Fan failure

- Blocked airflow

- Data-center cooling issue

- Dust

- High workload

## RAM/DIMM monitoring

Suppose HPE server mein 16 DIMMs installed hain.

iLO show kar sakta hai:

DIMM 1 → OK

DIMM 2 → OK

DIMM 3 → OK

DIMM 4 → Error

...

Aapko pata chal sakta hai ke particular memory module problem create kar raha hai.

Dell mein ye information iDRAC se mil sakti hai.

## Storage aur RAID monitoring

### HPE Storage Architecture

HPE servers commonly use HPE Smart Array controllers to manage storage.

HPE Server → Smart Array Controller → RAID Configuration → Physical Disks

Through the iLO interface, you can directly monitor both the overall storage health and detailed controller information.

### Dell Storage Architecture

Dell PowerEdge servers commonly use PERC (PowerEdge RAID Controller) controllers to manage storage.

Dell Server → PERC Controller → RAID Configuration → Physical Disks

Through the iDRAC interface, you can easily monitor both the storage array and the controller status.

## RAID monitoring

Is the process of regularly checking the health, performance, and status of your RAID (Redundant Array of Independent Disks) storage system so you know immediately if a hard drive is failing or has already failed?

Here is the English translation of your real-world RAID example:

### Real-World RAID Example

Suppose a Dell server has:

- 8 × SSDs installed.

RAID Controller:

- PERC configured as RAID 5

A disk failure occurs:

- Disk 1 → OK

- Disk 2 → OK

- Disk 3 → FAILED ❌

- Disk 4 → OK

- Disk 5 → OK

- ...

Monitoring and Notification:

- iDRAC displays a warning alert.

- The Systems Engineer is notified: "Disk 3 needs replacement."

Resolution:

- The faulty disk can then be hot-swapped/replaced, and the RAID array can automatically rebuild.

## Power Supply Monitoring

Enterprise servers often feature redundant Power Supply Units (PSUs).

Normal State:

- PSU 1 → AC → OK

- PSU 2 → AC → OK

If PSU 2 fails:

- PSU 1 → OK

- PSU 2 → FAILED ❌

System Impact and Alerts:

- The server typically remains running without interruption because PSU 1 continues to provide power.

- However, iLO/iDRAC triggers a critical warning alert.

- The Systems Engineer is notified that the PSU requires immediate replacement.

## Virtual Media — very important

Suppose aapko remote server par RHEL install karna hai.

Server Karachi mein hai.

Aap Islamabad mein hain.

Aap physical USB lekar Karachi nahi jana chahte.

Aapke laptop par:

RHEL ISO

available hai.

iLO/iDRAC ke Virtual Media feature se ISO ko remotely server ke virtual CD/DVD ke taur par attach kiya ja sakta hai.

Concept:

Your Laptop

|

RHEL ISO

↓

iLO / iDRAC

↓

Virtual CD/DVD

↓

Server

↓

RHEL Installation

Ye remote data-center operations mein extremely useful feature hai.

## Firmware Updates

Firmware is critical in enterprise server environments. Firmware is low-level software that is written directly to a hardware device's non-volatile memory to provide the permanent instructions that control how that hardware communicates and operates. Yes, exactly! Firmware is the low-level software embedded directly into a hardware device's non-volatile memory (like ROM, EEPROM, or Flash) that provides the essential instructions for how the hardware operates and communicates with other software.

Common Examples:

- BIOS / UEFI firmware

- RAID controller (PERC / Smart Array) firmware

- Network Interface Card (NIC) firmware

- Storage drive (HDD / SSD) firmware

- Baseboard Management Controller (BMC) firmware (iLO / iDRAC)

Management Capabilities:

- Through out-of-band management tools like iLO and iDRAC, administrators can inspect installed versions and apply firmware updates, depending on platform support and licensing tiers.

Workflow Example:

- Current BIOS: Version 2.10

- Available BIOS: Version 2.20

- The Systems Engineer can evaluate release notes, schedule a maintenance window, and execute the upgrade remotely.

## Logs — A Critical Part of Troubleshooting

Out-of-band management controllers like iLO and iDRAC continuously record hardware events into dedicated system event logs.

Timeline Example:

- 10:05 AM → PSU Warning ⚠️

- 10:15 AM → Fan Speed Warning ⚠️

- 10:20 AM → High Temperature Alert 🔥

- 10:25 AM → Sudden Server Reboot 🔄

Significance:

- If a server is crashing repeatedly, analyzing these sequential logs provides vital clues to identify the root cause (e.g., in this case, a failing fan caused overheating, which triggered an automatic thermal shutdown).

## Dedicated IP Addresses for iLO and iDRAC

By default, an out-of-band management controller maintains its own distinct IP address on the network, completely separate from the host operating system.

HPE Example:

- HPE Server (Host OS): 10.10.10.50

- iLO (Management Interface): 10.10.20.50

Dell Example:

- Dell Server (Host OS): 10.10.10.60

- iDRAC (Management Interface): 10.10.20.60

Network Architecture:

- In standard enterprise data center designs, the production network (used by customer-facing workloads) and the management network (used by administrators) are kept strictly isolated on separate subnets or VLANs for enhanced security and traffic segmentation.

## Management Network

In large-scale data centers, isolating management traffic from normal operations is an industry standard best practice.

Architecture Diagram:

## VMware with iLO / iDRAC

This scenario is critical for your upcoming Systems Engineer role.

Architecture Overview:

Failure Scenario:

- An ESXi host crashes or encounters a kernel panic (PSOD — Purple Screen of Death).

- vCenter Status: Displays as "Not Responding" or "Disconnected".

- Troubleshooting capabilities directly through vCenter become severely limited or entirely inaccessible.

## Out-of-Band Remediation via iLO:

- Launch the Remote Console (HTML5/IRC) to view the live physical display output.

- Inspect the host screen for a PSOD (Purple Screen of Death) and record the crash dump / backtrace error codes.

- Check the BIOS / POST diagnostics and verify real-time Hardware Health (e.g., failed memory DIMMs, storage controller errors, or thermal faults).

- Perform a graceful or hard Power Cycle / Reboot directly through the console if the host is completely hung.

(In a Dell-based VMware deployment, iDRAC performs the exact same operational role as HPE's iLO).

# Activity 1: Data Centre Components

## Rack Manifold

Figure. Rack Manifold

A Rack Manifold is a piping assembly installed in or near a server rack that distributes cooling liquid to multiple servers or cooling components and collects the heated liquid for return to the cooling system. It is commonly used in liquid-cooled data centers to ensure efficient and controlled heat removal from IT equipment.

## CDU

A Coolant Distribution Unit (CDU) is a cooling system component used in liquid-cooled data centers to circulate and regulate coolant flow between the cooling infrastructure and IT equipment, such as GPU servers. It typically uses pumps, heat exchangers, and monitoring controls to manage coolant circulation and transfer heat from the server cooling loop to a facility cooling system, helping maintain safe operating temperatures for the equipment.

Figure. Coolant Distribution Unit

A Coolant Distribution Unit (CDU) is a cooling system component used in liquid-cooled data centers to circulate and regulate coolant flow between the cooling infrastructure and IT equipment, such as GPU servers. It typically uses pumps, heat exchangers, and monitoring controls to manage coolant circulation and transfer heat from the server cooling loop to a facility cooling system, helping maintain safe operating temperatures for the equipment.

## How to Connect a CDU, Rack Manifold, and GPU Servers

## Step-by-step connection process

### Step 1: Connect the CDU to the cooling infrastructure

The CDU typically has two separate circuits:

- Facility water circuit: Connects the CDU to the building's approved cooling-water infrastructure.

- Technology cooling circuit (TCS): Circulates coolant between the CDU and the GPU servers.

In a typical liquid-to-liquid CDU, the heat exchanger transfers heat between these circuits without mixing their liquids.

### Step 2: Connect the CDU to the rack manifold

The CDU's technology cooling circuit has two main connections:

- Coolant Supply Outlet: Connect this to the rack manifold's supply inlet.

- Coolant Return Inlet: Connect this to the rack manifold's return outlet.

Use compatible pipes or hoses, fittings, valves, and quick-disconnect couplings specified by the equipment manufacturer.

### Step 3: Connect the rack manifold to the GPU servers

The rack manifold normally has a supply header and a return header, with branches for individual servers.

- Connect each server's designated coolant inlet to a supply branch.

- Connect each server's designated coolant outlet to a return branch.

- Verify that connector types, pressure ratings, coolant specifications, and flow requirements match.

Some GPU systems use blind-mate manifold connections, where the server connects directly to the manifold when inserted into the rack.

## CDU (Racks)

Figure. CoolIT CHx™200 is a rack-based CDU

CoolIT CHx™200 is a rack-based CDU, built for the needs of today’s most demanding HPC and enterprise server requirements. Capable of managing 200 kW of heat load in a remarkably small 4U of space, this system provides cooling for up to 200 servers.

## Liquid cooling hoses

Liquid cooling hoses transport heat-transfer fluids like water, glycol mixes, or deionized water through PC setups, AI servers, and data center thermal management systems.

# Firmware

Firmware is a special software that resides inside a hardware device and tells it how to function. In simple terms: hardware is a machine, and firmware provides that machine with the basic instructions on how to operate.

Example: Firmware in a Server

When you power ON a server:

- The server's firmware initializes the hardware.

- It helps check components like the CPU, RAM, and storage.

- The server boot process starts through the BIOS/UEFI.

- The server then loads the operating system, such as Linux or Windows Server

Real-life example: Just like a machine needs internal instructions to run, firmware provides the basic instructions to operate hardware.

Examples of Firmware

• BIOS/UEFI firmware: Helps boot the server.

• SSD firmware: Manages the internal operations of the SSD.

• NIC firmware: Controls the operations of the network interface card.

Remember: Firmware and the operating system (OS) are different. Firmware handles the basic operations of the hardware, while the operating system manages applications and system resources.

# What is a RAID Controller?

A RAID Controller is a hardware or software-based controller that configures and manages multiple physical drives to improve data protection, performance, or both.

If a server has 4 hard drives or SSDs installed, the RAID Controller can manage those drives according to a specific configuration.

Example: 4 drives in a server

Suppose 4 SSDs are installed in a server, and you want your data to remain available even if one drive fails. The RAID Controller organizes the data across the drives according to your selected RAID configuration.

## RAID 0 - Performance

Data is distributed across multiple drives. While this can improve performance, if even one drive fails, all data in the array can be lost

## RAID 1 - Mirroring

A copy of the data is also kept on another drive. If one drive fails, the data can remain available from the second drive.

## RAID 5 - Performance + Protection

A minimum of 3 drives are required. Data and parity are distributed across the drives. If one drive fails, the array can recover.

### How is Data Recovered When a Drive Fails? (Data Reconstruction)

This is the most magical feature of RAID 5. Parity is actually a mathematical calculation (based on what is called the XOR logic gate).

Let's understand this with a simple example:
Suppose you have two data blocks with the values 5 and 3. The system calculates their total (parity): 5 + 3 = 8.

- Drive A = 5 (Data)

- Drive B = 3 (Data)

- Drive C = 8 (Parity)

If Drive B fails and its data (3) is lost, the system looks at the remaining drives: 5 and 8. The computer immediately applies math: 8 - 5 = 3. The system instantly figures out that the failed drive contained the value 3.

Without turning off the computer, you can pull out the broken drive and insert a new one (a process called Hot Swapping). The system will automatically calculate the missing data using the remaining drives and rewrite it onto the new drive. This process is called a RAID Rebuild.

## Difference between Firmware and RAID Controller?

| Feature | Firmware | RAID Controller |
| --- | --- | --- |
| What is it? | Instructions that operate the hardware | A controller that manages drives in a RAID configuration |
| Function | Hardware initialization and device operations | Organizing data across multiple drives |
| Example | BIOS/UEFI firmware | Dell PERC, HPE Smart Array |
| Where is it located? | In the hardware's flash memory / chip | In the server motherboard, expansion card, or integrated controller |

RAID 10 (also known as RAID 1+0)

is a technology created by combining two different RAID levels RAID 1 (Mirroring) and RAID 0 (Striping). Its purpose is to simultaneously provide extreme speed (Performance) and excellent safety (Fault Tolerance). It is commonly referred to as a "Nested" or "Hybrid" RAID.

1. Minimum Drives Requirement

- A minimum of 4 hard drives is required to run RAID 10.

- In this setup, the number of drives must always be an even number, such as 4, 6, 8, or 10 drives.

### How Does It Work? (Mirroring + Striping)

In RAID 10, data is first mirrored (copied) and then those mirrors are striped (split into pieces). Let's understand this using an example of 4 drives:
Suppose you have 4 drives: Drive 1, Drive 2, Drive 3, and Drive 4.

First Step - RAID 1 (Mirroring):

  - Drive 1 and Drive 2 become a mirror pair. Whatever data goes to Drive 1, the exact same copy is saved on Drive 2.

  - Drive 3 and Drive 4 form the second mirror pair.

Second Step - RAID 0 (Striping):

  - The system breaks your files into pieces (blocks).

  - Piece A goes to the first mirror set (Drive 1 & 2).

  - Piece B goes to the second mirror set (Drive 3 & 4).

This way, you get very fast speeds due to RAID 0 because data is read/written to both sets simultaneously, and your data stays safe due to RAID 1.

### How is Data Protected When Drives Fail?

The fault tolerance of RAID 10 is excellent. This array can continue to run if at least 1 drive, and usually up to 2 drives fail but with one condition!

- If drives fail across different sets: Suppose Drive 1 (from Set 1) and Drive 3 (from Set 2) both fail at the same time, your system will keep running! This is because Drive 1's data is safe on Drive 2, and Drive 3's data is safe on Drive 4.

- If both drives in the exact same set fail: If Drive 1 and Drive 2 both fail at the same time, all your data will be lost because no backup remains for that entire block of data.

### Direct Comparison: RAID 10 vs. RAID 5

| Feature | RAID 5 | RAID 10 |
| --- | --- | --- |
| Minimum Drives | 3 Drives | 4 Drives |
| Storage Efficiency | Higher (N-1) - Only the capacity of one drive is used for parity. | Lower (N/2) - 50% of your total storage capacity is used for backup. |
| Read Speed | Good | Extremely Fast |
| Write Speed | Slower (Because Parity must be calculated) | Extremely Fast (No calculations needed, just a direct copy) |
| Rebuild Time | Very long (Puts high stress on remaining drives) | Very short and fast (Just a direct copy from one drive to another) |

When is RAID 10 Used?

Since this is an expensive setup (as you lose 50% of your storage space), it is not used in ordinary home PCs. Instead, it is used for large databases, heavily trafficked web servers, and financial trading platforms where thousands of transactions happen every second and speed cannot be compromised.

How it works?

# Storage

## SAN vs NAS

In server infrastructure, both SAN (Storage Area Network) and NAS (Network Attached Storage) are centralized storage solutions. The purpose of both is to store data, but the way both deliver data to servers is different.

First, understand a simple example: 
Imagine a company's data center has 20 servers. Each server has its own internal SSDs, but the company wants important data to be stored in a central location, so that multiple servers can access it as needed.
To solve this problem, either SAN or NAS can be used.

## What is NAS?

NAS stands for Network Attached Storage. NAS is a dedicated storage device or server that provides files to users and servers through a normal computer network, such as an Ethernet LAN. In Roman Urdu: You can think of NAS as a central file server connected to the network, where the company's documents, reports, images, videos, and backups are stored.

## Fig. Network Attach Storage

Look at this process step-by-step:

- The company employee opens the shared folder from their computer.

- The computer sends a request to the NAS through the network.

- The NAS checks the user's permissions.

- The NAS reads the requested file from its drives.

- The file reaches the employee's computer through the network.

### Which protocols are used in NAS?

- SMB/CIFS: Common in Windows computers and Windows file sharing.

- NFS: Common in Linux and Unix environments.

- HTTPS: Used in some NAS web interfaces and file-access applications.

### Where is NAS used?

- Company shared folders and documents.

- Department-wise files, such as HR, Finance, and IT.

- CCTV footage and media storage.

- Backup repositories.

- Home and office file sharing.

Real-life example: The Company’s HR department needs a shared folder to keep employee records. An HR folder can be created on the NAS, and access can be given only to authorized HR users.

## File vs. block vs. object storage

Each storage type can be used in different ways. For example, file storage for local file sharing, and block storage for high-performance applications. On the other hand, you can use object storage to store unstructured data like email, videos, image files, web pages, and sensor data produced by the Internet of Things (IoT)

## What is SAN?

SAN stands for Storage Area Network. SAN is a dedicated storage network that provides centralized block storage to servers. Think of SAN as a centralized storage system where the capacity of storage arrays can be assigned to servers in the form of logical disks or LUNs (Logical Unit Numbers). The server often sees this storage as a local disk, even though the actual drives are located in another machine or storage array.

Fig. Storage Area Network

How SAN works?

Step-by-step:

- The storage administrator configures the storage capacity in the SAN storage array.

- A logical volume or LUN is allocated to the server.

- The server's HBA (Host Bus Adapter) or network adapter establishes a connection with the SAN.

- The allocated storage appears to the server as a local disk.

- The operating system can create its own supported file system on that disk.

- Applications, databases, or virtual machines can read and write data on that storage.

Which protocols are used in SAN?

- Fibre Channel (FC): Dedicated high-performance storage networking technology.

- iSCSI: Transports SCSI block-storage commands over an IP/Ethernet network.

- Fibre Channel over Ethernet (FCoE): Technology to carry Fibre Channel traffic over an Ethernet infrastructure.

Where is SAN used?

- VM disks for VMware or other virtualization platforms.

- Enterprise databases.

- Business-critical applications.

- High-availability server clusters.

- Workloads that require predictable storage performance and centralized management.

Real-life example: A company has 10 virtualization servers. The administrator allocates storage from the SAN, and the virtual disks of the virtual machines are stored on it. This can make it easier to manage storage separately from the servers and allocate it as needed.

| Component | Core Function |
| --- | --- |
| Storage Array | Provides physical storage capacity and advanced storage services (snapshots, replication). |
| RAID / Storage Pool | Manages raw storage capacity, data protection, and performance scaling. |
| LUN (Logical Unit Number) | Presents a virtual block storage device/volume to the connected server. |
| HBA (Host Bus Adapter) | Provides physical Fibre Channel (FC) connectivity on the server side. |
| SAN Switch | Forwards and routes high-speed storage data traffic across the fabric. |
| Zoning | Restricts and secures node connectivity within the Fibre Channel (FC) fabric. |
| LUN Masking | Grants authorized hosts access to specific assigned LUNs at the storage controller level. |
| Multipathing | Manages and load-balances multiple redundant physical data paths for failover protection. |
| File System | Organizes and structures raw blocks into readable files and directories on the host. |
| Hypervisor | Configures and utilizes SAN storage as a shared datastore to host Virtual Machines (VMs). |

Physical Disks → Storage Pool → LUN → SAN Connectivity → Server Disk → File System → Application/VM Data

# SAN vs NAS important difference

NAS = File-Level Storage

NAS provides files and folders. ClienHts access files using protocols such as SMB or NFS.

SAN = Block-Level Storage

SAN provides storage to a server in the form of blocks. The server can manage its own file system.

## SAN vs. NAS Comparison Table

| Feature | SAN | NAS |
| --- | --- | --- |
| Full form | Storage Area Network | Network Attached Storage |
| Storage type | Block-level | File-level |
| Connectivity | FC, iSCSI, FCoE | Ethernet LAN |
| Common protocols | Fibre Channel, iSCSI | SMB, NFS |
| What does the server get? | Logical disk / LUN | Shared files and folders |
| Typical use | VM disks, databases, enterprise applications | Documents, shared folders, backups |
| Management | Usually requires specialized storage administration | Usually relatively simple |
| Cost | Generally higher | Generally lower |
| Performance | High-performance designs are possible | Depends on the network and NAS design |
| Scaling | Through storage arrays and SAN fabric | Through NAS capacity and network expansion |

SAN (Storage Area Network):

A Storage Area Network is a dedicated network that provides block-level storage to servers, commonly using Fibre Channel or iSCSI protocols.

NAS (Network Attached Storage):

Network Attached Storage is a storage device or system connected to a network that provides file-level access to users and servers through protocols such as SMB and NFS.