# Server+ Notes: Server Components vs. Desktop

Study notes from the IT Pro TV Server+ episode on server components.

## 1. Server vs. desktop

- A computer is a computer: both have a motherboard, CPU, RAM, storage, GPU and an enclosure.

- The two real differences are price and performance/reliability.

- Impact of failure is the key idea. A desktop outage affects one person, while a server outage can affect hundreds to hundreds of thousands of users and critical business assets.

## 2. Form factors

A form factor describes the shape, layout and size of the device.

| Type | Key points |
| --- | --- |
| Tower | Looks like a desktop and uses standard components. Easy to deploy and maintain, and desktop skills carry over. Least scalable. |
| Rack-mount | Horizontal, "rack and stack" in data centers. Better cable management, scalability, storage and security options. |
| Blade | Each blade is a self-contained module with its own CPU and RAM. Highest modularity and scalability. Drawback: it can't run alone, because the enclosure provides networking, backend storage connections and so on. |

## 3. CPU

- Multiprocessor: server boards have multiple sockets (the example showed 4), while desktops usually have one.

- Multicore and mostly 64-bit (32-bit is legacy).

- Must support virtualization: Intel VT and AMD-V.

- Cache is SRAM (small, fast, expensive) in levels L1, L2, L3. More cache means more performance and more cost.

### CPU families

- Current: Intel Xeon and AMD EPYC.

- Legacy: Itanium (Intel, IA-64) and Opteron (AMD).

### Sockets

- LGA (Land Grid Array): flat contacts on the CPU, pins in the socket, and a retention flap. Intel used it historically.

- PGA (Pin Grid Array): pins on the CPU, using ZIF (zero insertion force) sockets. AMD used it historically.

- Today, both Xeon and EPYC use LGA. This is likely exam material.

## 4. RAM

- RAM is volatile: it loses its data when power is lost.

- ECC (Error-Correcting Code): the module checks the stored data for errors and fixes them. It is a major server feature, and it adds cost.

### Two naming systems

- DDR (Double Data Rate) defines the chip and performance, and is set by JEDEC.

- PC-xxxx defines the finished module and its bandwidth, and is set by manufacturers.

### Generations

- Each DDR generation roughly doubles the data rate of the previous one.

- DDR3 lowers voltage versus DDR2.

- DDR4 lowers voltage again and allows much higher capacities.

- Know the pin counts and PC standards for each, as flashcard material.

## 5. Interfaces

### PCI (legacy)

- 32-bit bus, 33 MHz, 133 MB/s.

- Half-duplex (walkie-talkie style) and parallel.

- Parallel wiring causes crosstalk (EMI).

- Shared bus topology: only one device controls the bus at a time (bus mastering).

### PCIe (the successor)

- Serial and full duplex (simultaneous two-way), with less interference and much faster speeds.

- Multi-lane: x1, x2, x4, x8, x16, with x16 as the largest.

- Per-lane speeds double each version:

| Version | Per lane (approx.) |
| --- | --- |
| 1.0 | 250 MB/s |
| 2.0 | 500 MB/s |
| 3.0 | ~1 GB/s |
| 4.0 | ~2 GB/s |
| 5.0 | ~4 GB/s |

A card's total speed is the per-lane speed multiplied by the number of lanes. The instructor said the exam wants the rounded numbers, not the exact technical ones.

### USB

- Serial, 5 V power, up to 127 devices.

- 1.x is legacy and 2.0 is still widely used.

- 3.x went through confusing renames. The "1×1 / 2×2" naming refers to lanes aggregated, like PCIe.

- USB4 is the fastest at 40 Gbps.

### Other interface notes

- Server interfaces include adapter cards, external devices, and console cables (for example through a KVM for local management).

## 6. Corrections to the transcript

- The speakers say "gigabyte" for USB4 and PCIe 5.0, but USB4 is 40 gigabits/s (Gbps).

- The transcript says PCIe is "64-bit", but it's actually a serial lane-based link, not a bus width. Answer "serial, full duplex, multi-lane" on the exam.

- "Memory International Memory Association" is a transcription error for JEDEC.

- The "184 pins" figure was the original DDR (DDR1). Later generations use more pins on DIMMs, which you should confirm on your exam objectives.

- The USB speed figures for 2.0 and 3.x aren't in the transcript. Check them against your study guide.

## 7. Quick exam checklist

- The three form factors, and why blades need an enclosure.

- Why servers have multiple sockets, multicore CPUs, VT/AMD-V and big caches.

- Xeon vs. EPYC (current) and Itanium vs. Opteron (legacy), plus LGA vs. PGA.

- ECC, DDR vs. PC naming, and the DDR3 vs. DDR4 differences.

- PCI vs. PCIe (parallel vs. serial, half vs. full duplex, shared bus vs. lanes), plus the PCIe per-lane speeds.

- USB 5 V and 127 devices, and that USB4 is 40 Gbps.